	[image: image3.png]


   
	

	
	Safety Data Sheet



	

	Issue Date  01-Jan-2011  


	Revision Date:  24-Oct-2013  


	Version  1  




	CPCI-010 -  Microlube 



	
	Revision Date:  24-Oct-2013  



	_____________________________________________________________________________________________  



	

	1. IDENTIFICATION 

	   
	

	Product Identifier 
	

	Product Name 
	Microlube

	

	Other means of identification 
	

	SDS # 
	CPCI-010 

	UN/ID No 
	UN1760  

	   
	

	Recommended use of the chemical and restrictions on use 
	

	Recommended Use 
	Lubricant.  

	   
	

	Details of the supplier of the safety data sheet 
	

	Supplier Address

Cal Pac Chemicals, Inc

6231 Maywood Ave

Huntington Park, CA 90255  



	

	Emergency Telephone Number 
	

	Company Phone Number 
	(323) 585-2178  

	Emergency Telephone (24 hr) 
	Chemtrec 1-800-424-9300 (North America) 1-703-527-3887 (International)  

	

	2. HAZARDS IDENTIFICATION 

	   
	

	Appearance  Clear amber liquid  


	Physical State  Liquid  


	Odor  Mild detergent  



	   
	

	Classification 
	

	

	Skin corrosion/irritation 
	Category 1  Sub-category B  

	Serious eye damage/eye irritation 
	Category 1  

	Specific target organ toxicity (repeated exposure) 
	Category 1  

	   
	

	Signal Word

Danger  

	

	Hazard Statements

Causes severe skin burns and eye damage

Causes damage to organs through prolonged or repeated exposure  

	[image: image1.png]


[image: image2.png]





	

	Precautionary Statements - Prevention

Do not breathe dust/fume/gas/mist/vapors/spray

Wash face, hands and any exposed skin thoroughly after handling

Wear protective gloves/protective clothing/eye protection/face protection

Do not eat, drink or smoke when using this product  



	

	Precautionary Statements - Response 



	IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing

Immediately call a POISON CENTER or doctor/physician  



	IF ON SKIN (or hair): Remove/Take off immediately all contaminated clothing. Rinse skin with water/shower

Wash contaminated clothing before reuse  



	IF INHALED: Remove victim to fresh air and keep at rest in a position comfortable for breathing

Immediately call a POISON CENTER or doctor/physician  



	IF SWALLOWED: rinse mouth. Do NOT induce vomiting  



	

	Precautionary Statements - Storage

Store locked up  



	

	Precautionary Statements - Disposal

Dispose of contents/container to an approved waste disposal plant  



	

	Other Hazards

Toxic to aquatic life  



	

	Unknown Acute Toxicity

4.9% of the mixture consists of ingredient(s) of unknown toxicity  



	

	3. COMPOSITION/INFORMATION ON INGREDIENTS 

Chemical Name 

CAS No 

Weight-% 

(Z)-N-9-octadecenylpropane-1, 3-diamine 

7173-62-8 

5-10 

Ethoxylated Coconut Amine 

61791-14-8 

<5 



	

	4. FIRST-AID MEASURES 

	
	   
	

	First Aid Measures



	
	Eye Contact 
	Immediately flush with plenty of water. After initial flushing, remove any contact lenses and continue flushing for at least 15 minutes. Seek immediate medical attention/advice.  

	

	
	Skin Contact 
	Wash off immediately with plenty of water. Take off contaminated clothing. Wash contaminated clothing before reuse. Get medical attention immediately.  

	

	
	Inhalation 
	Remove to fresh air. Get medical attention immediately.  

	

	
	Ingestion 
	Do not induce vomiting. Drink plenty of water or milk immediately. Never give anything by mouth to an unconscious person. Get medical attention immediately.  

	

	Most important symptoms and effects



	
	Symptoms 
	May cause eye burns and permanent eye damage. Prolonged contact may even cause severe skin irritation or mild burn. Inhalation of fumes or acid mist can cause irritation or corrosive burns to the upper respiratory system, including nose, mouth and throat.  

	

	Indication of any immediate medical attention and special treatment needed



	
	Notes to Physician 
	Treat symptomatically.  

	

	5. FIRE-FIGHTING MEASURES 

	   
	

	Suitable Extinguishing Media

Water. Carbon dioxide (CO2). Dry chemical. Foam.  

	

	
	Unsuitable Extinguishing Media 
	Not determined.  

	

	Specific Hazards Arising from the Chemical

None known.  

	

	Protective equipment and precautions for firefighters

As in any fire, wear self-contained breathing apparatus pressure-demand, MSHA/NIOSH (approved or equivalent) and full protective gear. Cool containers exposed to fire with water.  Water or foam may cause frothing which can be violent, especially if sprayed into containers of burning hot liquid.

	

	6. ACCIDENTAL RELEASE MEASURES 

	   
	

	Personal precautions, protective equipment and emergency procedures  

	

	
	Personal Precautions 
	Use personal protective equipment as required.  

	

	
	Environmental Precautions 
	Prevent runoff to sewers, streams, and other bodies of water.  

	

	Methods and material for containment and cleaning up  

	

	
	Methods for Containment 
	Dike and contain spill.  

	

	
	Methods for Clean-Up 
	Pump liquid to salvage tank or soak up with absorbent material. Place in appropriate containers for disposal.  

	

	7. HANDLING AND STORAGE 

	   
	

	Precautions for safe handling



	
	Advice on Safe Handling 
	Wash thoroughly after handling. Use personal protection recommended in Section 8. Do not breathe dust/fume/gas/mist/vapors/spray. Do not eat, drink or smoke when using this product. Do not reuse this container. Thoroughly rinse all measuring devices. 

	

	Conditions for safe storage, including any incompatibilities



	
	Storage Conditions 
	Keep containers tightly closed in a dry, cool and well-ventilated place. Store locked up. Protect from freezing. Store away from incompatible materials. Keep out of the reach of children.  

	

	
	Incompatible Materials 
	Strong oxidizing agents. Strong mineral acids.  

	

	8. EXPOSURE CONTROLS/PERSONAL PROTECTION 

	   
	

	Exposure Guidelines 
	No exposure limits noted for ingredient(s)  

	   
	

	Appropriate engineering controls



	
	Engineering Controls 
	Ensure adequate ventilation, especially in confined areas.  

	

	Individual protection measures, such as personal protective equipment



	
	Eye/Face Protection 
	Wear safety glasses with side shields (or goggles).  

	

	
	Skin and Body Protection 
	Wear rubber or chemical resistant gloves to prevent skin contact. Long sleeve shirt and long pants.  

	

	
	Respiratory Protection 
	Ensure adequate ventilation, especially in confined areas.  

	

	
	General Hygiene Considerations 
	Handle in accordance with good industrial hygiene and safety practice.  

	

	9. PHYSICAL AND CHEMICAL PROPERTIES 

	   
	

	Information on basic physical and chemical properties



	Physical State 
	Liquid  

	Appearance 

Clear amber liquid  


	Odor 

Mild detergent  



	Color 

Amber  


	Odor Threshold 

Not determined  



	   
	

	Property 

Values  


	Remarks  • Method  



	pH 

>13  


	

	Melting Point/Freezing Point 

Not available  


	

	Boiling Point/Boiling Range 

Not available  


	

	Flash Point 

Not available  


	

	Evaporation Rate 

Not determined  


	

	Flammability (Solid, Gas) 

n/a-liquid  


	

	Upper Flammability Limits 

Not available  


	

	Lower Flammability Limit 

Not available  


	

	Vapour Pressure 

Not determined  


	

	Vapor Density 

Not determined  


	

	Specific Gravity 

1.06  


	(1=Water)  



	Water Solubility 

Completely soluble  


	

	Solubility in other solvents 

Not determined  


	

	Partition Coefficient 

Not determined  


	

	Auto-ignition Temperature 

Not available  


	

	Property 

Values  


	Remarks  • Method  



	Decomposition Temperature 

Not determined  


	

	Kinematic Viscosity 

Not determined  


	

	Dynamic Viscosity 

Not determined  


	

	Explosive Properties 
	Not determined  

	Oxidizing Properties 
	Not determined  

	Bulk Density 
	8.8 lbs/gal  

	

	10. STABILITY AND REACTIVITY 

	   
	

	Reactivity  

	Not reactive under normal conditions.  



	   
	

	Chemical Stability

Stable under recommended storage conditions.  

	

	Possibility of Hazardous Reactions

None under normal processing.  

	

	
	Hazardous Polymerization 
	Hazardous polymerization does not occur.  

	

	Conditions to Avoid

Keep out of reach of children.  

	

	Incompatible Materials

Strong oxidizing agents. Strong mineral acids.  

	

	Hazardous Decomposition Products

Amines. Nitrogen compounds. Carbon oxides.  

	

	11. TOXICOLOGICAL INFORMATION 

	Information on likely routes of exposure  

	

	
	Product Information 
	 

	

	
	Eye Contact 
	Causes severe eye damage.  

	

	
	Skin Contact 
	Causes severe skin burns.  

	

	
	Inhalation 
	Avoid breathing vapors or mists.  

	

	
	Ingestion 
	Do not taste or swallow.  

	   
	

	Information on physical, chemical and toxicological effects  

	

	
	Symptoms 
	Please see section 4 of this SDS for symptoms.  

	

	Delayed and immediate effects as well as chronic effects from short and long-term exposure  

	

	
	Carcinogenicity 
	This product does not contain any carcinogens or potential carcinogens as listed by OSHA, IARC or NTP.  

	

	
	STOT - repeated exposure 
	Causes damage to organs through prolonged or repeated exposure.  

	   
	

	Numerical measures of toxicity

Not determined  

	

	Unknown Acute Toxicity 
	4.9% of the mixture consists of ingredient(s) of unknown toxicity.  

	12. ECOLOGICAL INFORMATION 

	   
	

	Ecotoxicity 
	

	Toxic to aquatic life.  

	   
	

	Persistence/Degradability

Not determined.  

	

	Bioaccumulation

Not determined.  

	

	Mobility

Not determined  

	   
	

	Other Adverse Effects

Not determined  

	   
	

	13. DISPOSAL CONSIDERATIONS 

	   
	

	Waste Treatment Methods



	
	Disposal of Wastes 
	Disposal should be in accordance with applicable regional, national and local laws and regulations.  

	

	
	Contaminated Packaging 
	Disposal should be in accordance with applicable regional, national and local laws and regulations.  

	

	14. TRANSPORT INFORMATION 

	   
	

	Note 
	Please see current shipping paper for most up to date shipping information, including exemptions and special circumstances.  

	   
	

	DOT 
	 

	
	UN/ID No 
	UN1760  

	
	Proper Shipping Name 
	Corrosive liquid, n.o.s. ((Z)-N-9-Octadecenylpropane-1, 3-diamine)  

	
	Hazard Class 
	8  

	
	Packing Group 
	II  

	   
	

	IATA 
	 

	
	UN/ID No 
	UN1760  

	
	Proper Shipping Name 
	Corrosive liquid, n.o.s. ((Z)-N-9-Octadecenylpropane-1, 3-diamine)  

	
	Hazard Class 
	8  

	
	Packing Group 
	II  

	   
	

	IMDG 
	 

	
	UN/ID No 
	UN1760  

	
	Proper Shipping Name 
	Corrosive liquid, n.o.s. ((Z)-N-9-Octadecenylpropane-1, 3-diamine)  

	
	Hazard Class 
	8  

	
	Packing Group 
	II  

	
	Marine Pollutant 
	This material may meet the definition of a marine pollutant  

	   
	

	   
	

	

	15. REGULATORY INFORMATION 

	   
	

	International Inventories 
	

	 Not determined  

	   
	

	US Federal Regulations  

	   
	

	SARA 313

Not determined  

	   
	

	US State Regulations 
	

	   
	

	U.S. State Right-to-Know Regulations

Not determined  

	

	16. OTHER INFORMATION 

NFPA  
Health Hazards

2  

Flammability

1  

Instability

0  

Special Hazards

COR
HMIS  
Health Hazards

Not determined  

Flammability

Not determined  

Physical Hazards

Not determined  

Personal Protection

Not determined  

Issue Date 

01-Jan-2011  

Revision Date: 

24-Oct-2013  

Revision Note 

New format  

Disclaimer

The information provided in this Safety Data Sheet is correct to the best of our knowledge, information and belief at the date of its publication. The information given is designed only as a guidance for safe handling, use, processing, storage, transportation, disposal and release and is not to be considered a warranty or quality specification. The information relates only to the specific material designated and may not be valid for such material used in combination with any other materials or in any process, unless specified in the text.  

End of Safety Data Sheet  



	_____________________________________________________________________________________________  

	

	Page  1 / 7  

	


	_____________________________________________________________________________________________  

	

	Page  7 / 7  

	



